Operating Instructions

Suitable for the models with D81-03 of E400series and E401series

Safety precautions

Explanation of symbols

Refers to potentially dangerous situation that may cause serious injury or death .

DANGER

Refers to potentially dangerous situation that may cause injury or property losses .

NOTICE

FORBID

Refers to forbidded to do .

Refers to abide by the rules .

MUST BE ABIDED

The supplier shall not be liable for any loss arising from the failure to

comply with the following regulations

DANGER

1.This equipment is “I” type electric appliance, must be completed

connected to the grounding system.

2.If the supply cord is damaged, it must be replaced by the

manufacturer, its service agent or similarly qualified persons in

order to avoid a hazard.

3.This equipment is electric appliance, please do not be installed at

the place where can easily be splashed with water or high temperature.

Please install exhaust fan or ventilation opening to make sure there

is a good ventilation when using at Bathroom.

4.This appliance can be used by children below 3 years but require

constant observation by an adult in the immediate proximity of the

child.This appliance can be used by children aged from 3 years and above

and persons with reduced physical,sensory or mental capabilities or lack

of experience and knowledge if they have been given supervision or

instruction concerning use of the appliance in a safe way and

understand the hazards involved.Children shall not play with the appliance.

Cleaning and user maintenance shall not be made by children without

supervision.

5.The appliance will be permanently connected to the water pipe. Remove

the water pipe assembly except for installation and maintenance.

6.The portable residual current device(PRCD) is optional. The PRCD must

be reliably fixed.

1

Safety precautions

1. Do not insert or remove power plug with wet

hands. Otherwise there is a risk of electric

shock.

Do not damage the power cord and plug.

Otherwise there is a risk of electric

shock, short-circuit and fire.

FORBID

2.

3.

Do not use the loose socket.

Otherwise there is a risk of electric shock and fire.

4.

The power supply must be accorded with regulation.

Otherwise there is a risk of damage the product and fire.

5.

Do not disassemble, repair or reassemble the product.

Otherwise there is a risk of electric shock and fire.

6. Do not place a stove or a cigarette near the product.

Otherwise there is a risk of fire.

7.

Make sure not to splash water, detergents or urine on the

product and the power plug.

Otherwise there is a risk of electric shock,

damage the product and fire

8.Do not turn the product upside-down.

Otherwise this might cause product damage.

9.Do not bend or distort the inlet hose.

Otherwise there is a risk of water leakage.

10.The new hose-sets supplied with the appliance are to be

used and the old hose-sets should not be reused.

2

Technical Specification

E400-W011

E401-W011

MODEL

E400-F011

Rear Wash

Female Wash

Massage

●

●

●

Wash

Adjustable Water Temperatur

OFF/5 Steps（Default 3rd)

Adjustable Flow Rate

5 Steps（Default 3rd)

Adjustable Spray Position

Air Drying

5 Steps（Default 5th)

●

Warm Air

Heated Seat

Hygiene

Adjustable Air Rate

Adjustable Dryer Temperatur

Heated Seat

Adjustable Seat Temperature

Deodoriser

Nozzle Automatic Cleaning

Nozzle Manual Cleaning

Descaling

5 Steps（Default 3rd)

OFF/5 Steps（Default 3rd)

-

-

●

●

●

-

User Mode

-

D81-03

Remote Control

Night Light

Breathing Lamp

●

●

Soft Closing Seat And Cover

Body Knob

-

●

Rimless Toilet

●

Installation of the toilet

Wall-hung /Pedestal

Water Supply Connector

Rated Voltage

Power

G 3/8

AC220-240V，50/60HZ

1170W

Length Of Power Cord

Water Supply Pressure

Water Supply Temperature

Ambient Temperature

Protection Degree

Flow Rate

1.8m

0.07～0.75MPa(0.7～7.5Bar)

4～35℃

4～40℃

IPX4

350～650mL/min

31～39℃

Water Temperature

Wash Cycle Time

3 Minutes

Wash Device Heating Mode

Tank

Heater Power

Safety Device

Backflow Prevention

Speed Of Air

1200W

Thermal Cutoff Fuses\Liquid Level Switch\Thermostat

Air Gap of AB type

≥4m/S

Dryer Power

Dry Device Dry Cycle Time

250W

3 Minutes

Warm Air Temperature

35～50℃

Safety Device

Thermal Cutoff Fuses\Thermostat

Deodorization Mode

Deodorization Cycle Time

Heated Power

Diatom

Deodorization

Heated Seat

15Minutes

-

-

Seat Temperature

Safety Device

-

Frequency Range

2460MHz

Maximum RF Output Power

-10dBm

Remote Control Type of batteries

Number of batteries

Other Safety Device

Net Weight

3V CR2032

Two Pieces

Residual Current Devices

35Kg

Dimension

593*384*388 mm

Technical upgrade will be edited into the new version instruction without additional notice.

Please follow the real if it is different from the nominal in shape ,color and accessories.

3

Technical Drawing

E400-W011

4

Technical Drawing

E400-F011

Technical Drawing

E401-W011

5

Overview

Cover

Seat detector

Dryer

Breathing lamp

Night light

Rear wash

Female wash

Knob

Deodoriser

Seat

Toilet

The pictures of this instruction is for reference only .

Please subject to the actual product .

NOTICE

6

Remote Control

Female wash

Rear wash

Dry /

Stop

Steps

Increase flow /

Massage

Decrease flow

Increase

position /

Deodoriser

Decrease position /

Night light

Decrease

temperature /Shift

Increase

temperature /Shift

1.Before using,the remote control requires 2 batteries (type 3V CR2032)

2.Versions of Software : V1.0

NOTICE

(The software updates will not be notified. Please follow

the real remote control. )

7

Rear wash /Female wash

● Be seated

Press the "Wash" button on the remote control then it start to wash.

It will stop after 3 minutes . You can stop washing before this by

pressing the "Stop" button.

Rear wash

Female wash

OR

You can operate by rotating the knob . Rotate the knob in

the clockwise direction to activate"Female wash" and the

anticlockwise direction for "Rear wash". It will stop after 3

minutes and start to dry . You can stop washing before this

Female wash

Rear wash

by pressing the "Stop" button.

Note: While Rear wash or Female wash , it will wash at the fixed position

for about 10 seconds , and movable wash for about 18 seconds , then

regain to the fixed position until to the end .

Dry Operation

Be seated

●

●

It is not in the process of Rear or Female washing

Press the "Dry" button on the remote control then it

start to dry. It will stop after 3 minutes

OR

Press the knob then it start to dry . It will stop drying after 3 minutes

All the operation is just for the model with all features . If the model what you

bought has no the feature , please ignore the corresponding operation .

NOTICE

8

Massage Operation

In the process of Rear or Female washing

●

Keep pressing the "+ " or "-" button of temperature on the

remote control ,then press the "+" button of flow . Press it

again the wash will be on the diferent massage mode.

OR

Oscillating : when one indicator light is on

Pulsating : when two indicator lights are on

Oscillating + Pulsating : when three indicator lights are on

Stop Operation

●

In the process of Rear washing , Female washing or drying

Press the "Stop" button on the remote control it will stop washing or drying .

OR

Press the knob it will stop washing or drying

Adjusting water flow rate

● In the process of Rear or Female washing

Press the "+ " or "-" button of flow on the remote control to

adjust the flow rate . The indicator light of rate will be lighted up.

OR

You can operate by rotating the knob . The flow rate will

increase in the clockwise direction and decrease in the

anticlockwise direction .

Decrease

Increase

9

Adjusting the nozzle position

Default 5 th

●

In the process of Rear or Female washing

●

Press the"+ " or "-" button of flow position on the remote

control to adjust the nozzle position forward or backward.

Adjusting temperature

1.Adjusting water temperature

In the process of Rear or Female washing

●

Press "+ " or "-" button of temperature on the remote control

to adjust water temperature .

OR

Always press the knob and rotate it . The water

temperature will be increased in the clockwise direction

and be decreased in the anticlockwise direction .

Decrease

Increase

2. Adjusting air temperature

In the process of drying

●

Press the "+ " or "-" button of temperature on the

remote control to adjust air temperature .

OR

Always press the knob and rotate it .The air

temperature will be increased in the clockwise direction

and be decreased in the anticlockwise direction .

Decrease

Increase

10

Night light Setting

● Always on mode for the night light by default

Keep pressing the "+ " or "-" button of temperature on the

remote control ,then press the "-" button of position . Press

it again the night light will be on the diferent mode just as

"close","always on","smart".

OR

Close mode : when one indicator light is on

Always on mode : when two indicator lights are on

Smart mode : when three indicator lights are on

OR

Open the cover and seat , not be seated , then rotate the knob

in the anticlockwise direction. The night light will be on the

diferent mode just as "close","always on","smart".

NOTE: The "smart" mode refers that the night light will light up in a

dark environment and light off in a bright environment.

DeodorizeSetting

Smart mode for the deodorization by default . It is on when the user sits

●

down and it will be off after one minute when the user leaves .

If there is no operation of washing or drying during being seated , it will

be off after 15 minutes.

Keep pressing the "+ " or "-" button of temperature on the

remote control ,then press the "+" button of position .

Press it again the deodorization will be on the diferent

mode just as "close" or "smart".

OR

Close mode : when one indicator light is on

Smart mode : when two indicator lights are on

OR

Open the cover and be seated , without washing and

drying , then keep pressing the knob and rotate it in the

clockwise or anticlockwise direction. The deodorization

will be on or off .

11

Breathing lamp setting

● Always on by default

Keep pressing the "+ " or "-" button of temperature on

OR

the remote control ,then press the "Rear wash" button .

The breathing lamp will be lighted off . It will be lighted up

when pressing the "Rear wash" again .

OR

● Be seated

Open the cover and be seated , without washing and

drying , then keep pressing the knob for 6S .The

breathing lamp will be lighted off . It will be lighted up

again when pressing the knob for 6S .

Power StandbySetting

● In the state without seating

Keep pressing the knob for more than 6seconds , it will go

into power standby mode .

＞6s

Press again the knob for more than 6 seconds to restart it .

Sound setting

● The sound is on by default. There is a BEEP when the

bidet has received commands or signals

Keep pressing the "+ " or "-" button of temperature on the

remote control ,then press the "Female wash" button .Press

it again the sound will be on or off.

OR

off mode : when one indicator light is on

on mode : when two indicator lights are on

OR

Open the cover and seat .

Keep pressing the knob then rotate it. The sound is

on in the clockwise direction and off in the

anticlockwise direction.

12

Care and cleaning

General care and cleaning recommendations

●

Maintaining interval

Maintaining activity

Remove water drops with a soft ,lint-free cloth to avoid

limescale stains

Daily

Clean all surfaces and corners .Clean the spray nozzle if

required.See "Nozzle cleaning "

Weekly

Clean the WC ceramic appliance with a low acidic cleaning agents

Monthly

Every half year

Replace the filter if required.

Cleaning agents and tools

●

For E-bidet

For WC ceramic appliance

soft cloths ,lint-free cloths

soft cloths ,lint-free cloths

low acidic cleaning

agents .e.g. heavily

diluted citric acid.

mild cleaning agents

that are kind to the skin.

toilet brush.

Do not use the cleaning agents of strong acid and alkali to clean the E-bidet ;

Do not use the hard brush .e.g. toilet brush nylon brush to clean the WC ceramic

appliance ;

FORBID

Do not use the metal brush.

Self-cleaning the tank

Not used for more than 72 hours, when the user operates the

washing ,it will open the self-cleaning for about 3 minutes. All the

buttons are invalid during the self-cleaning and it will return to normal

after the self-cleaning.

NOTICE

13

Nozzle cleaning

● The nozzle will be cleaned before washing and after washing .

The operation of manual cleaning is as follows

Open the cover and seat then rotate the knob in the clockwise

direction .Then the nozzle will extend with cleaning water. It will

return after 5 minutes or press the " stop " button .

A

B

C

D

E

Replacing the filter

● The filter has a service life of half a year . This is depended on the environment

of water. The filter should be replaced when the wash flow is too small .

A

B

C

D

a new one

14

Drain the water tank

1.The water and power must be cut before you disassemble the E-Bidet

2.Do not invert or tilt it before you finish draining the water tank

'$1*(5

1.Hold the plug with the tipper , turn to

"open" , and pull out . It will start to drain .

2.After completing the drainage , insert

the plug and turn to "CLOSE" .

15

One-ON-One Setting

Install several electronic bidets in the same area, if you want each

●

remote control to control the specified electronic bidet, please complete

setting as per following steps.

1. Open both cover and seat and without seating. Set the night

light to the "close" model (just see the "Light setting ").

2. Keep pressing the "stop" button and "shift" button of adjust

temperature on the remote control at the same time for a few

seconds, then hear the "beep" from the product and meanwhile

the indicator light of first steps flicks.

3. Release the "stop" button, but keep pressing the "shift" button.

4.Press the "-" and "+" of the flow in order, and then release the

"shift"button. At this time, the lighted indicator is the current ID

code of product , and the 5th step light always flicks .

5.By pressing "-, +" buttons of the flow to change different

product ID code with corresponding indicator lights up. There

are 10 ID codes in total. It is on the first code by default and

the first step light lights up.

6.After selecting the product code ,press knob on the

bidet,then press the "stop" on the remote control.when

long "beep" sound heard,the setting is finished.

2

3

1

4

/

/

or

and

and

and

/

5

16

Replacing the batteries

The remotecontrolrequires 2 batteries (type3VCR2032).

●

●

When the level light flashes continuously , please replace the

batteries in time.

127,&(

1. Non-rechargeable batteries are not to be recharged

2. Rechargeable batteries are to be removed from the appliance before being charged

3.

Different types of batteries or new and used batteries are not to be mixed

4. Batteries are to be inserted with the correct polarity

5. The exhausted batteries are to be removed from the appliance and safely

disposed of

6. If the appliance is to be stored unused for a long period, the batteries

should be removed

7. The supply terminals are not to be short-circuited

B

Type: CR2032

A

C

17

ꢀTroubleshooting

Problem

Potential cause

Suggested action

1. The power supply may not be opened

2. The water supply may not be opened

3. The filter may be blocked

Please turn on the power

Please open the inlet water valve,or check if there is a water outage

Please replace a new filter .It has a service life of approx. half a year

Please sit on the seat and adjust sitting position

The washing can not work

4.Maybe not in the seating state

5.Maybe the cover is closed

Please open the cover and sit on the seat

Not used for more than 72 hours ,when the user operates the washing ,it will open the

self-cleaning for about 3 minutes. All the buttons are invalid during the self-cleaning

and it will return to normal after the self-cleaning

6.Maybe it is not used for more than 72 hours

1.The washing cycle is 3mins. The nozzle will stop

spraying after 3mins ,and then go back.

This is normal and please re-press the "wash" button to start washing again

Please check if there is a water outage

2.There may be some problems of water supply

The nozzle goes back suddenly

3.Maybe the seated sensor detects the user left the seat Please adjust sitting position

4.Maybe the cover is closing

Do not put down the cover when it is washing

Please increase the flow rate as per "adjusting water flow rate" of operating

instructions

1.The water flow rate may be set at low grades

2.The water supply pressure may be too low

3.The filter may be blocked

Please check and ensure the water supply pressure is ̱0.07Mpa

Please replace a new filter .It has a service life of approx. half a year

Please replace a new nozzle

Low water flow rate

4.The nozzle may be blocked

Please set the water temperature at high grade as per "adjusting temperature"of

operating instructions

Please set the water temperature at high grade as per "adjusting temperature"of

operating instructions,or decrease the flow rate as per "adjusting water flow rate" of

1.The water temperature may be set at low grades

2.Maybe the temperature of water supply is too low

Low water temperature

When start washing function,the nozzle will be self-

cleaning before coming out,and this will take some

The nozzle comes out slowly

This is normal

1.When start washing function,the nozzle will be self-

cleaning before coming out .When the washing is over This is normal

,also have this feature

The water flows out but the nozzle

do not come out

This is normal. Not used for more than 72 hours ,when the user operates the washing

,it will open the self-cleaning for about 3 minutes

2.Maybe it is not used for more than 72 hours

1. The power supply may not be opened

2.Maybe not in the seating state

3.Maybe the cover is closed

Please turn on the power

Please sit on the seat and adjust sitting position

Please open the cover and sit on the seat

The dryer can not work

Not used for more than 72 hours ,when the user operates the washing ,it will open the

self-cleaning for about 3 minutes. All the buttons are invalid during the self-cleaning

and it will return to normal after the self-cleaning

4.Maybe it is not used for more than 72 hours

Please set the air temperature at high grade as per "adjusting temperature"of

operating instructions

1.The air temperature may be set at low grades

2.The air heater can not work normally

1.Maybe in the bright environment.

Low air temperature

Please contact local distributor

When the night light is set at smart mode,the night light will not work when in the bright

environment.

Night light is always off

Please open the night light function and adjust the night light mode as per "Light

Setting" of operating instructions

2.The night light function may be closed.

1.Maybe in the dark environment.

When the night light is set at smart mode,the night light will be always on when in the

dark environment

Night light is always on

2.Something may be covered around the knob.

3.The night light may be set at always on mode

1.The nozzle goes back after washing 5 minutes.

Recover the obstructions around the knob

Please adjust the night light mode as per "Light Setting" of operating instructions

This is normal

The nozzle goes back automaticlly

when manual cleaning

The manual cleaning will be stopped if press the knob or press the "stop" button on

the remote control.

2.Maybe press the "stop" button

3.Maybe the seated sensor detects that the seat is

closing

Please do not put down the seat during the manual cleaning

1.Maybe there are not batteries in the remote control

2.Maybe the battery power is too low

Please install the correct batteries in the remote control

The remote control is not

functioning

When the battery power is too low the LED of grade will flash.Please replace the

battery

3.Maybe the code of the bidet and remote control is not Please set the code of the remote control to the correct one .See "One-On-One

the same

Setting"

There is a short BEEP when you are sitting down , and a

long BEEP when you leave. Also there is a short BEEP This is normal. You can turn off the sound. See "Sound Setting"

when the bidet has received commands.

There are BEEP sounds when the

bidet is working

Please shut off the power and turn on after one minute. If the bidet still cannot

work,please contact local distributor

The bidet can not work normally Maybe there is something abnormal

The light around the knob flash

twice with constantly BEEP

Please shut off the power and water supply,stop using the product,and then contact

local distributor

The water supply temperature may be too high

There may be something wrong with the product

The light around the knob keep

flashing

Please check if the electric power and water supply are normal,cut off the power and

restart the product. If the bidet still cannot work,please contact local distributor

18

NOTICE

Do not dispose of electrical appliances as unsorted municipal waste, use

separate collection facilities.

Contact you local government for information regarding the collection systems

available. If electrical appliances are disposed of in landfills or dumps,

hazardous substances can leak into the groundwater and get into the food

chain, damaging your health and well-being. When replacing old appliances

with new ones, the retailer is legally obligated to take back your old appliance

for disposals at least free of charge.

236E400-OCW-02

Revision No. 1.1

Date of Revised: 2025/01/10

19